Addis Ababa Science & Technology University

College of Architecture & Civil Engineering

Department of Civil Engineering

Engineering Mechanics (Statics) for Pre-Engineering Students

Work Sheet for Chapter-1

- Given the following three vectors V1=2i+j-k and V2=4i-3j and     V3=-3i+2j-5k.Determine the magnitude and direction of vector V given by V = V1 + V2 + V3

- For the coplanar system of vectors V1  and V2 shown below

- Determine the magnitude of vector sum V = V1 + V2 and the angle x which V  makes with the positive x-axis.

- Write V  as a vector in terms of the unit vectors i and j and then write the unit vector along vectorV .

- Determine the magnitude of vector difference V = V2 - V1 and angle x which V  makes with the positive x-axis.

- Determine the magnitude of vector difference V = V1 - V2 and the angle x which V  makes with x-axis.

- Find the value of k for which the vectors A = 4i-2j+8k and B = 12i-6j+kk are parallel.

- The resultant of the two forces, when they act at angle of 60 is 14N. If the forces are acting at right angles, their resultant is 136N. Determine the magnitude of the two forces.

- Three forces having a magnitude of 40KN, 15KN, & 20KN are acting at a certain point in the x,y plane. The angles made by 40KN, 15KN, & 20KN forces with the positive x-axis in CCW direction are 60, 120, and 240 respectively. Determine the magnitude and direction of the resultant force.

- Determine the magnitude and direction of vector V2 such that V= 2V1 + V2 acts along the positive z-axis having a magnitude of 20 from which vector V1 = 20i-15j+15k.